ACTIVITY: RAW MATERIAL HANDLING SYSTEM MAINTENANCE

- Objective	: -  Safe and quality maintenance of conveyor belts for optimum output

- Scope: - All conveyor belts at PID including Plant No.5, Plant No.8 and Slag Dryer Plant.

- Ref.: - Conveyor belts maintenance manual

- Responsibility: - Engineer- in- charge & Maintenance fitter on the job

PPE to be used	:

- Helmet, safety shoes, dust mask, hand gloves and  goggles, CO monitor full body safety harness

- Activity No 1		:	Conveyor belt changing

Activity No 2		: 	Joining belt by hot vulcanizing.

- Activity No 3		: 	Joining belt by Fasteners

- Activity No 4		: 	Changing of conveyor belt drum

- Activity No 5		: 	Changing of gear box

- Activity No 6		: 	Changing of motor

- Activity No 7		: 	Carrying roller/ bracket changing

- Activity No 8		: 	Working above conveyor belt

- Activity No 9		: 	Working below conveyor belt

- Activity No 10	            : 	Changing of gate

- Activity No 11	            :	Liner plate changing

- Activity No 12	            :	VT tube replacement and vibration setting

- Activity No 13	            :	Replacing/cleaning of dust suppression nozzles

- Activity No 14	            :	VF/VC liner plate changing

- Activity no 15                   :           Belt structure changing and repairs

- Activity no 16                   :           Change over of drive of CB 5 conveyor

- General guidelines for belt joining:  Inter Plant Standard Steel Industry IS 10288-1082

Aspect- Impact

| Scrap generation | Resource Depletion |
| --- | --- |
| Oil spillage | Land contamination |
| Oil traced waste generation | Land contamination & Resource Depletion |
| Dust generation | Air pollution |
| Fumes | Health |
| Fire | Air pollution SP-42 |

Hazards identified

Physical Hazard

Fall of oil in ears, eyes, mouth

Temperature

Mechanical Hazard

Trapping between two objects,

Fall of material like hammer, tools, slung items, bolts, rollers wedges, etc,

Fall of person from platform,

Fall of rotating part like coupling,

Entanglement in between drums, belts, rollers, etc.

Impact of moving / slung items

Failure of welding hook

Cut from tool/ belt cutting knife

Flying off of gas cutting and welding sparks

Human behaviour aspect of operators: Operator nature, alcoholism, casual approach, back pain, horse play  & non usage of PPE?s

Electrical hazard      -    Shock

Chemical hazard  -    Vulcanizing compound splashing onto the eyes, Gas poisoning , fire

# Activity No 1		:	Conveyor belt changing

- Take clearance from Production dept. to carry out the job

- Put the concerned belt in manual mode of operation and position the joint of the belt (preferably at the tail end return side).

- Take electrical shutdown of the equipment

- Cut the old belt, join one end of the new and old belt with minimum 6 belt fasteners and to the other end of old belt tie a manila rope. If the belt is having Gravity Take up (GTU), then take load of GTU using chain block.

- Temporarily clear electrical shutdown to lay the new belt and keep the belt in manual mode. Electrician should be available in panel room while laying belt with motor drive.

- Instruct all workmen on the job not to put hand in between rollers, drum and conveyor belt and only pull the belt.

- While laying the belt, position persons in the following way:

- 2 persons near the new roll, to loosen the belt.

- 4 persons to pull the old belt rope side.

- 2 persons on either side of conveyor belt to move along with temporary joint, to ensure that the joint does not get stuck while laying the belt.

- One person near the ON/OFF switch and another at the tail end for communication while laying the belt.

- 4 persons should pull the belt and, after the belt is tensioned on the return side, put on the drive.

- Don’t keep the drive running for long and ensure that the new belt  is getting released from it’s roll

- Once the laying is over take electrical shutdown of the belt

- Remove the fasteners of the joint, required number of carrying rollers and brackets to clamp the belt.

- Cut sufficient amount of belt to make the joint and use 2 clamps each on either side of joint. Lock clamps of one side firmly to conveyor structure.

- Use pulling and lifting machine to tension the belt and after tensioning put another clamp and lock it firmly to the conveyor structure carrying side.

- Vulcanize the joint – Refer activity no: 2

- After required curing time of the belt, remove the clamps and release the chain block of GTU. Ensure all the clamps from the belt are removed and all brackets and rollers put back. Rotate input coupling by hand, to reconfirm that all the clamps have been removed.

- Clear the shutdown and start the belt in manual mode to take trials.

- Hand over the belt to Production dept.

Activity No 2	: Joining the belt by hot vulcanizing.

- Take clearance from Production dept.

- Put the belt in manual mode and position the joint of the belt.

- Remove the required number of carrying rollers and brackets to clamp the belt.

- Clamp the belt with 2 angle clamps each on either side of the joint (Total 4 nos of angle clamps). In case there is more than one joint for the belt, then ensure that the distance between them is not less than 6 meters.

- Start preparing the joint by cutting the belt parallel to each other. (Mark with perpendicular scale/ right angle).

- Fix the heating equipment and keep for required time till the temperature reaches 150 deg Celsius.

- After required curing time of belt and cooling of heating equipment, remove the clamps and release the chain block of GTU. Ensure that all the clamps from the belt have removed and all the brackets and rollers put back. Rotate input coupling by hand to reconfirm that all the clamps have been removed.

- Clear the shutdown and start the belt in manual mode to take trials.

- Hand over the belt to Production dept.

- Care should be taken to avoid splashing of vulcanizing compound onto the eyes.

HOT VULCANISING QUIPMENT ASSEMBLED AND BELT JOINT KEPT FOR HEATING

# Activity No 3		: 	Joining belt by fasteners

- Take clearance from Production dept.

- Put the belt in manual mode of operation and position the joint of the belt on top side such that it is convenient for fastener joint.

- Take electrical shutdown

- Remove the required number of carrying rollers and brackets to clamp the belt.

- Clamp the belt with 2 angle clamps each, on either side of the joint (Total 4 nos of angle clamps).

- Start preparing the joint by cutting the belt parallel to each other using right angle(Fig 2)..      (Mark with perpendicular scale). Only skilled people should handle belt cutting instruments.

- Cut the belt in right angle (Fig 3) and touch both the cut edges so that there is no gap

- Keep fastener upper plate along the belt joint at equidistance of 15mm between each fastener as shown in photos below. Mark holes using pen / pencil and punch out holes on both sides of the belt joint (Fig 4,5,6,7,8,9 & 10).

- Insert fastener bolts inside the lower plate and lock those bolts in its place by retainer plate locking. Pass these prepared set of fastener through holes punched, head facing downwards. Ensure hole punch is kept on top half of circle marked with fasteners. This is done as belt will get pulled inwards after fasteners are put. Place upper plate on bolts. Tighten fastener nuts with the help of ratchet and cut the extra thread of the bolt 2mm above nut, with a hacksaw(Fig 12,13,14,15,16,17,18 & 19)

- Hammer the cut side to flatten it. (Fig 20) Cut the belt on both ends of joint to prevent it from getting stuck at side of conveyor structure while in operation (Fig 21)

- Remove angle clamps from the belt. Put back all the removed brackets and rollers. Rotate input coupling by hand to reconfirm that all the clamps have been removed.

- Lower primary, secondary and internal scrapper so that clamps does not get open after strucking with the scrappers

- Clear the shutdown and start the belt in manual mode of operation, to take trials. Adjust skirt rubbers if necessary.

- Hand over the belt to Production dept.

|  | NOS OF CLAMP USED FOR JOINT |  |
| --- | --- | --- |
| BELT WIDTH | 1 1/2 INCH CLAMP | 2 INCH CLAMP |
| 800 MM | 19 | 18 |
| 650 MM | 16 | 15 |
| 500 MM | 12 | 11 |

CONVEYOR BELT JOINING STEPS

Material required for belt joining with fasteners:

- FASTENERS

- MEASURING TAPE

- WOODEN PLANK

- HAMMER

- SCREW DRIVER

- RIGHT ANGLE

- HACKSAW FRAME WITH BLADE

- NUT TIGHTENING RATCHET

12.  SMALL HOLLOW PIPE DIA 10 MM (ID)

11.  BELT CUTTING KNIFE

10. HOLE PUNCH 10 MM

9. WATER IN SMALL BOWL

(Fig 1)

BELT BEING MARKED AND CUT USING RIGHT ANGLE

(Fig 2)

BELT  AFTER RIGHT ANGLE CUT

(Fig 3)

FASTENERS KEPT IN LINE FOR MARKING HOLES

(Fig 4)

FASTENERS KEPT IN LINE SPACED AT EQUIDISTANCE OF 15MM EACH

(Fig 5)

HOLES MARKED WITH PEN

(Fig 6)

FASTENERS REMOVED AFTER  HOLE BEING MARKED

(Fig 7)

ENSURE HOLE PUNCH IS KEPT ON TOP HALF OF THE CIRCLE THAT WAS MARKED EARLIER USING FASTENER.

(Fig 8)

ENSURE HOLE PUNCH IS KEPT ON TOP HALF OF THE CIRCLE THAT WAS MARKED EARLIER USING FASTENER.

(Fig 9)

HOLE BEING PUNCHED WITH HOLE PUNCH BY DEEPING IN WATER THE HOLE PUNCH

(Fig 10)

FASTENERS BEING INSERTED IN HOLE FROM BOTTOM OF BELT

(Fig 11)

FASTENERS BEING INSERTED IN HOLE FROM BOTTOM OF BELT AND ADJUSTED WITH SCREW DRIVER

(Fig 12)

FASTENERS TOP PLATE BEING INSERTED FROM TOP SIDE OF FASTENER

(Fig 13)

FASTENERS SEEN FROM BACKSIDE OF BELT WIL LOOK LIKE THIS

(Fig 14)

FASTENERS TOP PLATE  AND NUT BEING FITTED FROM TOP SIDE

(Fig 15)

NUTS BEING TIGHTENED USING RATCHET

(Fig 16)

FASTENERS AS SEEN FROM TOP SIDE AFTER BOLT TIGHTENING

(Fig 17)

EXTRA PORTION OF BOLT THREADED PORTION BEING HALF CUT AFTER NUT IS TIGHTENED

(Fig 18)

HALF CUT BOLT IS THAN BENT AND REMOVED TO SAVE TIME IN CUTTING FULL THREADED PORTION OF BOLT.

(Fig 19)

TOP CUT PORTION OF BOLT BEING HAMMERED TO FLATTEN THE CUT PORTION AND PREVENT NUT FOM COMING OUT

(Fig 20)

BELT IS CUT ON SIDE AS SEEN IN PHOTO TO AVOID ENTANGLEMNT OF BELT IN CONVEYOR STRUCTURE WHILE IN OPERATION

(Fig 21)

1. BOTTOM PLATE

2. RETAINER PLATE

3. TOP PLATE

(Fig 22)

# Activity No 4	: Changing of conveyor belt drum

- Take clearance from Production dept.

- Take electrical shutdown.

- Remove the required number of carrying rollers and brackets to clamp the belt.

- If the belt is having Gravity Take Up (GTU), take load of the GTU using 3T chain block.

- Loosen screw take up to minimum.

- Put 2 clamps each on carrying and return side of belt on either side of the drum to be changed.

- Put sling on shaft on either side of the drum.

- Remove top half of plummer block and lift the drum using chain block or hydra, whichever is convenient.

- Place the new drum in position and fix the plummer block halves.

- Remove all clamps from belt. Put back all the removed brackets and rollers. Also release load on chain block from GTU.

- Clear shutdown and start the belt in manual mode of operation to take trials.

- Hand over the belt to Production dept..

# Activity No 5	: Changing of gear box

- Take clearance from Production dept.

- Take electrical shutdown.

- Decouple input and output side coupling.

- Sling the gearbox through the holes provided on the topside.

- Lift, using hydra or chain block, whichever is convenient.

- Place the overhauled gearbox in position and align it..

- Couple input and output side coupling.

- Clear electrical shutdown and start the belt in manual mode to take trials.

- Hand over the equipment to Production dept.

# Activity No 6	: Changing of motor

- Take clearance from Production dept.

- Take electrical shutdown.

- Decouple motor from gearbox.

- Sling the motor through hook provided on top side.

- Lift using hydra or chain block whichever is convenient as per work instruction WI/MAINT/12

- Place the overhauled motor in position and align it..

- Couple motor and gearbox.

- Clear the electrical shutdown and start the belt in manual mode to take trials.

- Hand over the equipment to Production dept.

# Activity No 7		: 	Carrying roller/ bracket changing

- Take clearance from Production dept.

- Take electrical shutdown (LOTO) of belt if the belt can be stopped without disturbing the process operation. If not than take temporary shutdown of belt as mentioned in shutdown procedure SP42.

- Keep electrician in panel room to put off the belt. Once confirmation is taken from electrician than pull the pull cord and press emergency push button at site.

- Barricade the area below, if the job is to be carried out at height. Keep one person below the roller changing area to give alert for persons moving below the work area.

- Lift the belt above the roller, manually, remove the roller and insert the new roller.

- For bracket changing, remove the roller and then the bracket. Position the bracket and put the rollers back.

- Fix back the safety nets and tie at every 2 mtr to avoid fall hazard of rollers

- Clear the electrical shutdown, release pull cord and emergency push button and take trials.

- Hand over the equipment to Production dept.

# Avtivity No 8		: 	Working above conveyor belt

- Take clearance from Production dept.

- Take electrical shutdown.

- Before starting the job, cover the belt below with GI sheet or MS plate of required length.

- Keep water in bucket / hose connected and fire extinguisher ready at site if gas cutting / welding is carried out above belt area. Also depute 1 person to keep a watch on the belt.

- Once the job is completed, ensure that all scrap generated is shifted from site and GI sheet or MS plate is removed.

- Clear the electrical shutdown.

- Hand over the equipment to Production dept.

# Activity No 9		: 	Working below conveyor belt

- Take clearance from Production dept.

- Take electrical shutdown of the conveyor belt to avoid falling of material or rollers from top.

- Electrical shutdown is mandatory for working below GTU and removal of barricades below GTU for cleaning purpose.

- Ensure availability of proper net below the conveyor

- Clear electrical shutdown.

- Hand over the equipment to Production dept.

# Activity No 10		: 	Changing of Gate

- Take clearance from Production dept.

- Take electrical shutdown of the conveyor belt.

- Put off the air supply and remove the hose connections from the cylinder.

- Disconnect the cylinder shaft from gate.

- Remove the side bolts to dismantle the gate.

- In case you are gas cutting, follow the procedure of working above conveyor belt.

- Place the new gate in position and put the bolts.

- Connect gate with cylinder shaft.

- Connect the pneumatic hoses to cylinder.

- Put on air supply and take trials in manual mode.

# Activity No 11		:	Liner plate changing

- Take clearance from Production dept.

- Take electrical shutdown of the equipment, wherever possible.

- In case you are working above a  belt then

- Before starting the job, cover belt below with GI sheet or MS plate of required length.

- Ensure water and fire extinguisher at job site. Also depute 1 person to keep watch on the belt.

- Once the job is completed, ensure that all scrap generated is shifted from site and the GI sheet or MS plate is removed.

- If working at height, barricade the area below.

- If the bin has load cell, ask instrumentation dept. to remove the same and give clearance.(direct earthling has to be given to the equipment)

- Remove the damaged liner plate by gas cutting.

- Fix new liner plate with countersunk screws and also tack weld for additional safety. In case of load cell, give clearance to instrumentation to fix load cell.

- Clear electrical shutdown and take trials in manual mode, wherever possible.

- Hand over the equipment to Production dept.

- Activity No 12	:	VT tube replacement and vibration setting

- Take production clearance & ensure that the bunker is empty.

- Take electrical shutdown. & disconnect   the equipment.

- Fix chain pulley block above VT at designated place. Check the structural stability of the hooking point

- Remove the vibrator actuator assembly with care.

- Lift VT & lower VT on platform then get it shifted out of shed. Take precautions to avoid hitting of the assembly with other equipments.

- Get the new VT on the platform & lift it with the help of a chain pulley block.

- Put the  holding bracket  spring  in position & release the chain pulley block

- Fix the vibrator actuator assembly. Check the plates for any damage and replace, if needed.

- Check the gap between the coil & the plates. Adjust the gap to the required vibration level by adjusting the back adjusting bolt

- Tighten the adjusting bolt to ensure fitting of the plates

- Ask electrical dept. for connection of motors / coils ; Take trials in manual mode

- Check the material flow for the desired rate.

- Hand over the equipment to Production dept

Activity No 13	:	Replacing/cleaning of dust suppression nozzles

- Take  clearance from Production dept., for stopping the system

- Close the air & water line valves

- Remove the nozzle cap.  Take precautions to avoid falling of nozzle & nozzle cap on the moving belt.

- If the nozzle & nozzle cap holes are found to choked, remove the air & water line pipes and clean the nozzle using a fine pin.

- if nozzles are damaged, replace the old nozzle with new one .

- Check nozzles by opening the water line.

- Re-fix  the  nozzle cap on nozzle assembly and connect  with quick release coupler  and tubing

- If the water sprays remain on in idle condition, check for solenoid valve choke up.

- After cleaning solenoid valve , start the system

- Activity No 14	:	VF/VC liner plate changing

- Take clearance from Production dept.

- Take electrical shutdown of the equipment, wherever possible.

- In case you are working above a belt, cover belt below with GI sheet or MS plate of required length, before starting the job.

- Ensure the availability of water and other fire extinguisher at job site. Also depute 1 person to keep a watch on the belt.

- Once the job is completed, ensure that all scrap generated is shifted from site and GI sheet or MS plate is removed.

- While working at a height, barricade the area below.

- If the bin has a load cell, ask instrumentation dept. to remove the same and give clearance (direct earthing has to be given to the equipment)                                                  Remove the damaged liner plate by gas cutting.

- Fix new liner plate with countersunk screws and also tack weld for additional safety.  Give clearance to instrumentation dept. to fix the load cell.

- Clear the electrical shutdown and take trials in manual mode, wherever possible.                          Hand over the equipment to Production dept

- Activity no 15:  Belt structure changing and repairs

1. Take electrical shut down of the belt and take work permit

2. Cut the belt at the joint and pull the belt backwards tying with manila rope.

2.  Inform & Remove all cables (electrical and instrumentation dept) and pipe line from both the sides of structure if any

3. Divide the belt structure into appropriate pieces so that it can be lifted easily with the crane / Hydra.

4. Provide support to the belt structure so that it don’t collapse on cutting

5. Dismantle the structure with the help of crane / Hydra using proper tools, certified slings and tackles.

6. Place the new pieces align it, bolt the structure and do the welding of joints.

7. Then remove the head end and tail end structures and place the newly made structure.

8. Conveyor belt to be laid on the structure and join it either by cold lagging/ hot lagging.

9. Place the side walk way grating and do the welding.

10. Simultaneously electrical can place the cables.

11. Clear the shutdown take trial for its alignment

12...Monitor the belt for smooth operation.

13. If repairs of structure is to be done than cut the member which is to be replaced one by one. Ensure not more than one member of structure is cut at a time to maintain the stability of structure.

14. If in doubt consult design engineer for clarification.

15. If member cannot be removed individually than lapping to be done with MS plate, ISA or ISMC.

.

- General guidelines for belt joining:  Inter Plant Standard Steel Industry IS 10288-1082

- Activity no 16                   :           Change over of drive of CB 5 conveyor

- Take clearance from production department (Work Permit) from the shift Superintendent / In charge and obtain the shutdown clearance from Electrical Department.

## 2.  For carrying out changeover of CB-5 MOTOR-1/BF2 TO  CB-5 MOTOR-2 /BF2  & VICE VERSA    following procedure to be carried out.

- Take clearance from production for stopping the CB-5/BF2 drive which is  running.

- After the CB-5 drive is put off , take shutdown of both the CB-5/BF2 motors ( ie CB-5 MOTOR-1/BF2 and CB-5 MOTOR -2/BF2) for carrying out the changeover.

3    Remove coupling guard on the stand by gearbox.

4  Connect the coupling of the stand by gearbox. Replace the bush if found to be worn-out.

5    De couple the gearbox which is in service.

6   Clamp the other side coupling half on the drum shaft to avoid rubbing over the shaft  with decoupled gear box.

7    Check the oil level of the gearbox in service before handing over. If require top up the oil.

8   After completion of the job restore all safety guards, take electrical clearance and complete the restoration of equipment as per shut down procedure. Inform production control room engineer after the changeover so that he can do the necessary changeover of drive selection in PLC/HMI.( ie CB-5 MOTOR-1 /BF2  to  CB-5 MOTOR-2/BF2 or vice versa)

9   Observe the gear box for abnormal sound and request shift engineer to monitor the  gear box performance for next 24 hours.

10  Use material handling facility as per WI/MAINT/12 wherever required.

11  Follow housekeeping procedure as per instruction specified by WI/MAINT/91.

Do’s

- Use dust mask in dusty area.

- Use water to settle down the dust. (As per condition of site)

- Carry CO monitor and monitor gas level in working area

- Take shutdown of equipment before start of job

- Carry out housekeeping after completion of the job as per work procedure WI/MAINT/91

- Take shutdown of both the CB-5 both motors ( ie CB-5 MOTOR-1/BF2 and CB-5 MOTOR -2/BF2) for carrying out the changeover.

DO NOT:

- Work without taking shutdown of equipment.

- Lean over the working platforms.

- Put hand or keep tools on conveyor belt .

- Cross over the conveyor belts

Do’s

- Carry out the lubrication of the gearbox coupling as per work procedure WI/MAINT/93

- Carry out housekeeping after completion of the job as per work procedure WI/MAINT/91

- Inform and take clearance from instrumentation dept before carrying out any welding job on equipment / bunkers fitted with load cells.

Amendement Record

| Date | Manual Section Ref. & Para | Brief details of Revision | New Rev. |
| --- | --- | --- | --- |

| Prepared By: Area Engineer | Reviewed & Issued By: Management Representative | Approved By: Mechanical Head |
| --- | --- | --- |
| Signature | Signature: | Signature: |
| Review Date: 12.12.22 | Review Date: 12.12.22 | Review Date: 12.12.22 |