WORK INSTRUCTIONS FOR CAST HOUSE NON-DRAINABLE RUNNER PRACTICE

Responsibility: Furnace Incharge

Identified Hazards:

- Contact with hot metal & slag.

- Splashing of liquid hot metal in contact with wet/damp tools

- Slipping of personnel in the runner

- Nonuse of PPE & not following WI

- Improper house keeping

- Inadequate local lighting

- Inhalation of fumes

- Burns due to contact with hot metal & slag.

- Runner getting puncture

- Using rice husk containing moisture

List of non-standard tools used in cast house

  - In house prepared lancing hook

  - In house prepared pawda

  - In house prepared manual rammer

- Instruction during Normal Operational:

Unauthorized operation or repair of any equipment is a punishable offence.

Pl follow the below mentioned instruction/procedure for non-drainable runner operation

- Do not cross over the main runner or dam area with or without grating support & do not throw any material across the main runner

- Physical access control to prevent unauthorized entry in all critical hazardous area to ensure only trained and authorized people have access to these areas, in addition runner to be guarded during shutdown.

- Any activity planned on tuyere no 1 and 8, compulsorily runner needs to be drained off.

- While working on the main runner platform, does not face back towards the runner to avoid accidental slippage into the main runner.  Don’t climb on the Main runner / wall at siphon pit when the main runner is filled with hot metal.

- When dam preparation is going on, ask others to avoid working on main runner.

- Use poking rod drill bit for poking.

- Metal/slag level should be lowered immediately after closing of the cast so that it is not more than the dam height by agitating with bamboo or lancing pipe and cover hot metal in the runner and siphon pit with rice husk.

- Metal head and slag head levels should be maintained, metal head should be lower than 100 mm with respect to slag head.

- No personnel is allowed to work in the ladle placement area and below the main runner including repairing of spouts.

- Tools to be used in cast house which come in contact with liquid hot metal, should not be wet /damp (i.e. crow bars, hand shovels, poking rods & lancing pipes etc.) to avoid splashing

- Before opening of the cast always ensure top layer of metal pool on both side of the skimmer plate is in molten state, if not remove the crust and check whether skimmer hole is clear or not by inserting long poking rod. If require use bamboo to clear the hole.

- Before opening cast ensure ladle to be placed exactly below the bypass runner spout.

- Ensure tap hole is opened slowly till the hot metal crosses the skimmer plate, once the hot metal crosses the metal dam only than should the taphole be poked to clear the opening.

- Thermocouple temperature should be monitored hourly basis in each shift by Furnace in charge. If any deviations from normal operation (drastic increase in temperature) is observed and it is to be highlighted. If temperature increase beyond 800 degree C, drain the runner and observe the erosion pattern and repair if required. Responsibility: Furnace in Charge.

- Check always the launder cooling outlet air temp & flow is there

- Ensure that no shell water is falling on the main runner.

- Check the tap hole jacket for leakages.

- Draining of the Main Runner.

After major repairs, run the trough straight upto 20000 to 25000 THM (Continuous running).

  - Draining of the main runner is to be done only for the following reasons.

    - When furnace planned / unplanned shutdown hours increase beyond 4 to 5 hours.

    - When the furnace is on colder side (metal temperature is coming below 1300°C.

  - For planned shutdown, runner cooling fan to be stopped just two casts before the last cast. Ensure that runner side wall’s temperature does not cross 500°C. If side wall temperature crosses above 500°C, start the runner cooling fan immediately

  - In case bypass hole is to be opened and metal is drained it should be ensured that skimmer hole and bypass hole is clear.

  - Ensure wood pieces, bamboo lancing system and manpower with all PPE is ready.

  - Divert the ladle to second ladle when the ladle is less than half. As soon as cast is closed, lance the bypass hole and drain the runner.

  - In case bypass hole is opened and metal is drained it should be ensured that skimmer hole and bypass hole is clear.

  - Insert wood to maintain connectivity in both the holes.

  - Stop the runner cooling fan immediately after / before draining the main runner.

  - Due to any reason if runner is drained, and runner condition is good no repair is required, we should keep the runner hot by heating runner always with BF gas heater or LPG heating or firewood.

- Taking Metal in Main Runner:

- Do not start runner cooling fan for the first two casts after furnace start-up. If runner side wall’s temperatures increases beyond 500°C, you have to start the runner cooling fan

- Heat the main runner continuously till the runner wall temperature reaches to 500 to 550°C.

- Ensure skimmer plate hole is clear with all slag and metal.

- Bypass runner hole to be cleaned of slag/metal thoroughly and plugged with runner mass, then rammed packed the hole and heated till runner mass is set properly (hard).

- Close the bypass hole with runner mass.

- Heating of the bypass hole with firewood etc.

- Open the cast slowly.

- Ensure the runner cooling fan is running after runner side wall temperature reaches above 350°C

- Main Runner preparation during the furnace blow in / furnace revival etc.:

  - Drain the furnace main runner as per the draining furnace if furnace is cold.

  - Clean the main runner, skimmer plate and bypass hole.

  - Make the runner drainable runner.

  - If hot metal is to be taken into EL, close skimmer’s hole with sand and divert the metal into EL.

- Midterm repairs:

Check for the availability of the material well in advance of the major repairs (5 tons). Check all equipment and machinery for their proper functioning one day prior to the repair.

Dismantling of trough for minor repair:

- Ensure the trough is drained out completely.

- Keep the trough in idle condition for 1 hour.

- Use water – air spray / cooler for faster cooling.

- Please follow the work instruction as per WI/PROD/08-H.

NON-DRAINABLE RUNNER PRACTICE- BF2

Responsibility: Furnace Incharge

Identified Hazards:

- Contact with hot metal & slag.

- Splashing of liquid hot metal in contact with wet/damp tools

- Slipping of personnel in the runner

- Non use of PPE & not following WI

- Improper house keeping

- Inadequate local lighting

- Inhalation of fumes

- Burns due to contact with hot metal & slag.

- Runner getting puncture

- Glare of metal

List of non-standard tools used in cast house

  - In house prepared lancing hook

  - In house prepared pawda

  - In house prepared manual rammer

- Instruction during Normal Operational:

Unauthorized operation or repair of any equipment is a punishable offence.

Pl follow the below mentioned instruction/procedure for non-drainable runner operation

- Do not cross over the main runner or dam area with or without grating support & do not throw any material across the main runner

- While working on the main runner platform, does not face back towards the runner to avoid accidental slippage into the main runner.  Don’t climb on the Main runner / wall at siphon pit when the main runner is filled with hot metal.

- When dam preparation is going on, ask others to avoid working on main runner.

- Use poking rod drill bit for poking.

- Metal/slag level should be lowered immediately after closing of the cast so that it is not more than the dam height by agitating with bamboo or lancing pipe and cover hot metal in the runner and siphon pit with rice husk.

- Metal head and slag head levels should be maintained, metal head should be lower than 100 mm with respect to slag head.

- No personnel is allowed to work in the ladle placement area and below the main runner including repairing of spouts.

- Tools to be used in cast house which come in contact with liquid hot metal, should not be wet /damp (i.e. crow bars, hand shovels, poking rods & lancing pipes etc.) to avoid splashing

- Before opening of the cast always ensure top layer of metal pool on both side of the skimmer plate is in molten state, if not remove the crust and check whether skimmer hole is clear or not by inserting long poking rod if require use bamboo to clear the hole.

- Before opening cast ensure ladle to be placed exactly below the bypass runner spout

- Ensure tap hole is opened slowly till the hot metal crosses the skimmer plate, once the hot metal crosses the metal dam only than should the taphole be poked to clear the opening.

- Thermocouple temperature should be monitored hourly basis in each shift by Furnace in charge. If any deviations from normal operation (drastic increase in temperature) is observed and it is to be highlighted. If temperature increase beyond 500°C, drain the runner and observe the erosion pattern and repair if required. Responsibility: Furnace in Charge.

- Check always the launder cooling outlet air temp & flow is there

- Ensure that no shell water is falling on the main runner.

- Check the tap hole jacket water is also leaking.

- Ensure that runner cooling fan is running always.

- Draining of the Main Runner.

After major repairs, run the trough straight upto 20000 to 25000 THM (Continuous running).

  - Draining of the main runner is to be done only for the following reasons.

    - When furnace planned / unplanned shutdown hours increase beyond 4 to 5 hours.

    - When the furnace is on colder side (metal temperature is coming below 1300°C).

  - For planned shutdown, runner cooling fan to be stopped just two casts before the last cast. Ensure that runner side wall’s temperatures does not cross 500°C. If side wall temperature crosses above 500°C, start the runner cooling fan immediately.

  - In case bypass hole is to be opened and metal is drained it should be ensured that skimmer hole and bypass hole is clear.

  - Ensure wood pieces, bamboo lancing system and manpower with all PPE is ready.

  - Divert the ladle to second ladle when the ladle is less than half. As soon as cast is closed, lance the bypass hole and drain the runner.

  - In case bypass hole is opened and metal is drained it should be ensured that skimmer hole and bypass hole is clear.

  - Insert wood to maintain connectivity in both the holes.

  - Stop the runner cooling fan immediately after / before draining the main runner.

  - Due to any reason if runner is drained, and runner condition is good no repair is required, we should keep the runner hot by heating runner always with BF gas heater or LPG heating or firewood.

- Taking Metal in Main Runner:

- Do not start runner cooling fan for the first two casts after furnace start-up. If runner side wall’s temperatures increase beyond 500°C, you have to start the runner cooling fan.

- Heat the main runner continuously till the runner wall temperature reaches to 500 to 550°C.

- Ensure skimmer plate hole is clear with all slag and metal.

- Bypass runner hole to be cleaned of slag/metal thoroughly and plugged with runner mass, then rammed packed the hole and heated till runner mass is set properly (hard).

- Close the bypass hole with runner mass.

- Heating of the bypass hole with fire wood / coke etc.

- Open the cast slowly.

- Ensure the runner cooling fan is running after runner side wall temperature reaches above 350°C

- Main Runner preparation during the furnace blow in / furnace revival etc. :

- Furnace under revival:

  - Drain the furnace main runner as per the draining furnace if furnace is cold.

  - Clean the main runner, skimmer plate and by pass hole.

  - Fill with hot sand for entire main runner, siphon pit etc.

  - Ensure that upper hole of the skimmer plate’s hole is cleaned with runner mass.

  - If hot metal is to be taken into EL, close upper skimmer’s hole with sand and divert the metal in to EL.

- Furnace under Blowing in:

  - Clean the main runner, skimmer plate and by pass hole.

  - Fill with hot sand for entire main runner, siphon pit etc.

  - Ram the main runner, siphon pit with runner mass. Thickness of the ramming mass could be 100 mm.

  - Heat the runner mass till runner mass completely heated.

  - Ensure that upper hole of the skimmer plate’s hole is cleaned with runner mass.

  - If hot metal is to be taken into EL, close upper skimmer’s hole with sand and divert the metal in to EL.

- Midterm repairs:

Check for the availability of the material well in advance of the major repairs (8 – 9 tons).Check all equipment and machinery for their proper functioning one day prior to the repair.

Dismantling of trough for minor repair:

- Ensure the trough is drained out completely.

- Keep the trough in idle condition for 1 hour.

- Use water – air spray / cooler for faster cooling.

- Measure the wear pattern of the trough using erosion measurement tool and record (at each 1 mt at both sides).

- Erosion Measurement Tool.

- Whenever runner is drained the following are need to be observed.

- Check for any cracks are found.

- Check for any rat hole are found (rat hole are those which are more than 200 mm deep penetrating through the sidewall castable towards back up).

- Dent (Irregular oval shaped erosion at the side wall lining).

- Metal penetration and slag penetration.

- Erosion on side walls and especially at Metal impact area.

- Check the dam area & height if is eroded

- Measure the slag and metal head depth.

- Use at least 2-3 numbers heavy duty pneumatic breaker for dismantling. Use air distribution for operating the pneumatic breakers.

- Oxidized material should be removed completely.

- All deep cracks, rat holes should be removed completely.

- All the metal penetration /slag infiltration should be removed completely. Remove the metal / slag jam completely from side walls.

- Remove the worn out refractory in such a way that minimum 100 mm gap is maintained between the shuttering plate and residual lining for side wall casting.

- In case of metal penetration, remove the metal penetration by V groove cutting. All the metal penetration should be removed completely.

- For cracks more than 12 mm, or cracks with dent holes or cracks with rat holes area to be removed by V groove cutting.

- For all the repairs, Elephant foot should be made at side wall bottom corner.

- After dismantling, clean the entire trough and remove fine material by compressed air so that no loose material is left out. Ensure that this activity is done in the direction of exhaust fan, so that all the fines are sucked up by the fan to the dust collector. All unnecessary (unauthorized) persons should be evacuated and remaining workmen should wear dust mask / adequate dust protection equipment.

- Measure the trough temperature with laser gun.

Placement of Shuttering:

- Use 3 mm  to 5 mm plate with proper supporting arrangement  for shuttering to avoid any to avoid any deformation.

- Shuttering should be fixed for built up side wall (as per actual condition) & areas where V groove has been made.

- If there is any erosion is seen at the skimmer block, skimmer shuttering needs to be fixed.

Casting:

Removing of Shuttering Plate:

Additional Skimmer plate hole (incase of skimmer plate hole chockes).

- Prepare one more skimmer hole in the skimmer plate only 350 mm from the top of skimmer plate before start-up of the furnace and close with hydrous clay. Size of the hole should not exceed 100 mm.

- If due to any reason, main skimmer plate chokes, skim of the metal from the sphion pit to just below the additional skimmer plate hole and open the additional skimmer plate hole.

- Before opening the additional skimmer plate hole, prepare / increase the slag dam height and metal dam height.

- Run the casting from the additional i.e. top skimmer plate hole.

- Agitate the skimmer plate’s bottom hole during the casting with bamboo during casting only.

- This agitation should be done till bottom skimmer plate gets open.

- After bottom skimmer plate hole gets open close the top additional skimmer plate hole

- Agitation of bamboo to be done before cast opening after bottom hole is open to keep bottom hole in open condition for two cast minimum.

| Prepared By: Head – Production PID I | Reviewed & Issued By: Management Representative | Approved By: Head – Pig Iron Division |
| --- | --- | --- |
| Signature: | Signature: | Signature: |
| Date: 15.07.2022 | Date: 15.07.2022 | Date: 15.07.2022 |

| Amendment Record |  |  |  |
| --- | --- | --- | --- |
| Revision date | Manual Section ref. and para | Brief details of revision | New Revision No. |
| 12.07.2021 | Procedure for cast house non-drainable runner practice | Point no 11,12,20 (Hazard) | 11 |
| 15.07.2022 | Procedure for cast house non-drainable runner practice | Change in format | 12 |