WORK INSTRUCTIONS FOR OPERATING BF2  DUST EXTRACTION UNIT & BAG HOUSE

Responsibility:  Cast house in charge/ Control room in charge / Utility in charge

Identified Hazards:

1.	Contact with lime powder, graphite dust

2.	Contact with hot hood causing burns

3.	Contact with dust, lime & fumes

4.	Fire & explosion

5.	Electric shock

6.	Nonuse of PPE

7.	Improper house keeping

8	Inadequate local lighting

9.      Entrapment of person while unloading graphite from bags

10.    Human error

Significant Aspect:

1. Emission of graphite

2. Consumption of compressed air

Sequence of operation

Safety Interlocks & Tripping:

Compressed Air pressure low <4.S Bar: System will Trip & Stop sequence will activate automatically.

Motor & ID fan's DE & NOE bearing temperatures reaches >= 100°C: System will Trip & Stop sequence will activate automatically.

Level Switch High (LSH) activation: System wi ll Trip after 30 minutes of high alarm &

Stop sequence will activate automatically.

. / VFD Tripping: System will Trip & Stop sequence will activate automatically.

. / DP value reaches >150 MMWC: System will Trip after 30 minutes of high alarm &

Stop sequence will activate automatically.

. / ID fan & Motor DE & NOE Vibration >=7 mm/sec: System will Trip & Stop sequence will activate automatically.

. / RAL Tripping: System will Trip after 30 minutes of high alarm & Stop sequence will activate automatically.

Troubleshooting:

Level Switch High (LSH) Activate:

  - Identify the LSH from P&ID or the MIMIC screen and check physically at site.

Rectify the same and re-start the system.

VFD Tripped under Voltage Dip:

  - Reset the VFO by pressing the reset button on VFO feeder panel and restart the system.

Compressor not starting in Auto Mode after tripping or emergency push button on:

  - Reset the compressor feeder in feeder panel and press the start button in compressor panel itself.

Pulsing Pressure not maintained/ not ok:

  - Check for any leakages through SOV's

  - Check individual headers for pressure sustenance and identify the header for SOV leakages id entification

  - Attend the leakages and restart the compressor. 	Any line leakages to attend to maintain pressure re.

RAL's tripping:

  - Check for jamming in RAL's

  - If yes. Clear the jam and restart the e system

Drag Chain Conveyor (DCC) Jamming & Tripping:

  - Check for jamming in DCC

  - If yes. Clear the jam and restart the system

| Prepared By: Head – Production PID I | Reviewed & Issued By: Management Representative | Approved By: Head – Pig Iron Division |
| --- | --- | --- |
| Signature: | Signature: | Signature: |
| Date: 10.07.2023 | Date: 10.07.2023 | Date: 10.07.2023 |

| Amendment Record |  |  |  |
| --- | --- | --- | --- |
| Revision date | Manual Section ref. and para | Brief details of revision | New Revision No. |
| 12.07.2021 | Procedure for SHUTDOWN OF BLAST FURNACE - II | Point no 7,20,35 | 14 |
| 15.07.2022 | Procedure for Shutdown of Blast Furnace -II | Change in format | 15 |