WORK INSTRUCTIONS FOR OPERATION OF BF#2 SCRUBBER UNIT

Responsibility: Shift Superintendent/Cast house In charge/Furnace In charge

Identified Hazards:

- BF Gas leakage/Poisoning

- Fire and Explosion in the gas line

- Contact with hot water

- Fall of person

- Human behavior-Nonuse of PPE

- Human Behavior-Improper house keeping

- Human behavior-Not carrying CO detector

- Burns due to contact with hot water/surface

Significant Aspects:

- Usage of water/spillage of water

- BF Gas leakage

- Fire & Explosion

Procedure for Scrubber operation:

View_ BF2 Furnace side

Access to Scrubber Unit_27 MTR side

- Taking Scrubber In line:

    - If required, Purge the Scrubber unit thoroughly with steam

    - Close all individual relief valves and drop HBS water seal.

    - Start BF2 GCS pump, ensure all individual water valves are kept open.

GCS Inlet Valve

    - Maintain water flow at 50-60 M3/Hr in scrubber unit.

    - Ensure the water drain tank is always kept full.

Cleaning valves

Cleaning valves

    - After taking scrubber in line continuous monitoring of parameters from PLC to be ensured such as water flow, BF Gas DP, Moisture % and dust % level. As shown in below pic.

- Isolating the Scrubber Unit:

      - Water seal the HBS/GPH.

      - After water seal ensure continuous water flow.

      - Stop water flow to the scrubber unit by closing individual valves.

      - If required steam purging to be carried and open subsequent relief valves.

| Prepared By: Head – Production PID I | Reviewed & Issued By: Management Representative | Approved By: Head – Pig Iron Division |
| --- | --- | --- |
| Signature: | Signature: | Signature: |
| Date: 10.07.2023 | Date: 10.07.2023 | Date: 10.07.2023 |

| Amendment Record |  |  |  |
| --- | --- | --- | --- |
| Revision date | Manual Section ref. and para | Brief details of revision | New Revision No. |
| 15.07.2022 | Work instructions for Scrubber unit | Process change | 01 |